REQUIREMENTS SPECIFICATION Functional, Non-Functional, and Safety Requirements sen-TapIn — NFC Access & Attendance Terminal (v0.1.0 baseline, MicroPython)

| Document Title | REQUIREMENTS SPECIFICATION |
| --- | --- |
| Document Number | REQ-SPEC-003 |
| Revision | 1.0 |
| Status | DRAFT |
| Date | 26 June 2026 |
| Project | sen-TapIn — NFC Access & Attendance Terminal |
| Product Version | v0.1.0 (baseline) — MicroPython build (variant: +mp) |
| Lead Engineer | Supun Akalanka Sriyananda (Owner) |
| Tech Engineer | Ravindu Madanayaka |
| Supporting Engineer | Chenuli Athukorala |
| Affiliated Companies | Sen-Net, Arpico Energy, Soran |
| Related Docs | HDD-003, TP-003, CS-003 (planned) |

# 1.  Purpose and Scope

This document defines what the sen-TapIn terminal must do, how well it must do it, and the constraints it must respect. It is the anchor for the rest of the project: the Hardware Design Document (HDD-003) realises these requirements in hardware, the Test Plan (TP-003) verifies them, and the Coding Standard (CS-003) governs how they are implemented.

Requirements are grouped into Functional (REQ-F), Non-Functional (REQ-NF), and Safety (REQ-S). Each carries a stable identifier so other documents can reference it precisely. This revision covers the v0.1.0 baseline — the MicroPython build (Layer 1). The same requirements are intended to carry forward unchanged to the Pico SDK / C build (v0.2.0), which targets feature parity; requirements that belong to the later hardening stage (v0.3.0) are marked as deferred or optional.

Note — the interaction model is explicit-mode, two-button. Direction of travel is chosen by the operator using two momentary buttons (IN and OUT) rather than inferred automatically. Pressing a button arms that direction; a card presented within the arm-timeout window is logged in the armed direction. This choice removes any dependence on stored per-user state, so no direction state can be lost or corrupted across a power cycle. It is realised in hardware by HDD-003 §2.6 and §3.3.

# 2.  Functional Requirements

| ID | Requirement | Verified by |
| --- | --- | --- |
| REQ-F-001 | The terminal shall read the UID of a presented ISO 14443A card (Mifare Classic / NTAG) via the PN532 over I2C. | TP-003 functional |
| REQ-F-002 | Pressing the IN button shall arm IN mode and pressing the OUT button shall arm OUT mode; the armed mode shall be shown on the OLED (primary) and reflected by the RGB idle tint (secondary). | TP-003 functional |
| REQ-F-003 | If no card is presented within the arm-timeout window (default 10 s) after arming, the terminal shall return to neutral idle and log nothing. | TP-003 functional |
| REQ-F-004 | On a card read while a mode is armed, the terminal shall look up the UID against the allow-list on the SD card and produce a Granted or Denied result. | TP-003 functional |
| REQ-F-005 | The terminal shall signal the result: on Granted, RGB green + confirmation tone + relay(LED) pulse + OLED "Granted" with resolved name; on Denied, RGB red + error tone + OLED "Denied". | TP-003 functional |
| REQ-F-006 | The terminal shall append one record to the attendance log on the SD card for every decided authorisation event — Granted and Denied alike — per the schema in Section 5. | TP-003 logging |
| REQ-F-007 | All logged timestamps shall be read from the DS3231 real-time clock. | TP-003 logging |
| REQ-F-008 | The allow-list shall be a human-readable file on the SD card mapping UID to name, editable without special tools. | Inspection |
| REQ-F-009 | When not armed, the OLED shall display the current time (from the RTC) and a prompt to select a direction. | TP-003 functional |
| REQ-F-010 | A Denied result shall not actuate the relay, but shall still be recorded in the log (result = DENIED). For an unknown card, the record stores the raw UID with the name field marked "unknown". | TP-003 functional |
| REQ-F-011 | (Optional / stretch) The terminal may serve the attendance log as a read-only web page over Wi-Fi. Not required for v0.1.0 completion. | TP-003 optional |
| REQ-F-012 | The same UID re-presented within the duplicate-suppression window (default 5 s) after a Granted event shall be ignored and not re-logged. Suppression applies to grants only; repeated Denied attempts are each logged. | TP-003 functional |

# 3.  Non-Functional Requirements

| ID | Requirement | Verified by |
| --- | --- | --- |
| REQ-NF-001 | From card read to user-visible feedback shall be no more than 1 second under normal operation. | TP-003 latency |
| REQ-NF-002 | The terminal shall perform all core functions (read, decide, feedback, log) with no network connection. | TP-003 functional |
| REQ-NF-003 | The attendance log shall be append-only, with each record flushed to the SD card such that an unexpected power loss loses at most the in-progress record. | TP-003 durability |
| REQ-NF-004 | Time shall be maintained by the DS3231 and survive power cycles via its battery backup; drift shall be within the device's rated accuracy. | TP-003 timekeeping |
| REQ-NF-005 | The firmware shall use exactly the pin assignments and I2C addresses defined in HDD-003 §3. | Build inspection |
| REQ-NF-006 | The IN and OUT buttons shall be debounced so a single physical press arms exactly once. | TP-003 functional |
| REQ-NF-007 | If the SD card or the RTC is unavailable at start-up, the terminal shall display the fault on the OLED and shall not enter normal operation — it shall not read cards or actuate outputs until the fault clears. | TP-003 fault |

# 4.  Attendance Log Schema

Every decided authorisation event (REQ-F-006) is one record appended to the log file on the SD card. Records are written in a human-readable, append-only format (one line per event). The fields below are the minimum required set; additional diagnostic fields may be appended without breaking earlier records.

| Field | Description | Example |
| --- | --- | --- |
| timestamp | Date and time of the event, from the DS3231 (ISO 8601). | 2026-06-26T08:14:03 |
| uid | Card UID as read by the PN532, hex. | 04A2B3C4 |
| name | Resolved name from the allow-list; "unknown" if the UID is not listed. | R. Madanayaka |
| direction | Armed direction at the time of the tap. | IN |
| result | Authorisation outcome. | GRANTED |

Forward note — denied-UID logging is revisited in the hardening stage. Logging Denied events (REQ-F-010) means the SD card accumulates the UIDs of cards that tried and failed. Because a UID is not a secret and the log is a plain file (see REQ-S-002), this is acceptable for the v0.1.0 learning baseline and is useful for debugging and audit. In the hardening stage (v0.3.0), storing UIDs in the clear should be revisited — for example by hashing logged UIDs — so that this decision is deliberate and traceable rather than accidental.

# 5.  Safety Requirements

Safety requirements derive from the terminal's role and its deliberately limited baseline scope. Each maps to a hardware constraint in HDD-003 and will be tracked in the Risk Register.

| ID | Requirement | Related constraint |
| --- | --- | --- |
| REQ-S-001 | In v0.1.0 the relay shall drive only a low-voltage indicator LED; no mains voltage and no inductive load shall be connected. | HDD REQ-HW-003 |
| REQ-S-002 | UID-based authorisation shall not be relied upon as a secure access-control mechanism; sen-TapIn v0.1.0 is a learning and attendance baseline, not a security device. | HDD REQ-HW-002 |
| REQ-S-003 | On any unrecoverable fault the terminal shall default to a non-granting state: the relay shall be de-energised and no unlock shall occur (fail-closed). | HDD REQ-HW-003 |

# 6.  Constraints and Assumptions

The hardware platform is fixed by HDD-003 (Raspberry Pi Pico 2 W, DS3231, PN532 / DFR0231-H, SSD1306 OLED, MicroSD module, WS2811, two mode buttons, relay driving an indicator LED). The pin map and I2C addresses are frozen by HDD-003 §3 and REQ-HW-001, and the same requirements are intended to be met by both the MicroPython (v0.1.0) and Pico SDK / C (v0.2.0) builds without hardware change. Default timing values (arm timeout 10 s, duplicate-suppression window 5 s) are working values and may be tuned during bring-up without a requirement change, provided the change is recorded. A physical lock, its power supply, and network features (Wi-Fi log delivery beyond the optional view, NTP synchronisation, remote logging) are deferred to later stages and specified in their own documents.

# 7.  Traceability

Each requirement here is referenced by identifier in the documents that realise or verify it: HDD-003 (hardware and pin map), TP-003 (verification and pass criteria), and CS-003 (implementation conventions). Safety requirements additionally map to hardware constraints REQ-HW-002 and REQ-HW-003 and will be carried into the Risk Register. When a requirement changes, the referencing documents shall be reviewed for impact. The Lead Engineer owns this specification and approves changes to it.